SITE FIRE SAFETY PLAN

Operating Businesses should ensure that only persons that have received sufficient training and are competent to undertake construction fire safety assessment and planning complete this document.

Training for directly employed personnel may provide the necessary level of competency to assess and plan the fire safety on the majority of Persimmon sites.

More complex, timber frame sites could require more experienced persons i.e. Fire Safety Engineer however this resource should be decided at the pre-start meetings.

The Persimmon Site Fire Safety Plan is intended to identify the controls to be implemented to ensure the removal and/or reduction in the risk of fire occurring on site.

In the first instance Table’s 1 & 2 of this document must be completed to confirm the type of properties (traditional and/or timber frame) being built.

This in turn will indicate which sections of the plan (Part A/B) require assessing in order to identify suitable sufficient and adequate fire safety controls..

Generic Fire Safety Plans can often be sufficient however it may be necessary to complete a separate Part B for each apartment block/run of 3+ storey town houses.

Details of the Part A and B assessments must be included in the Construction H & S Plan and updated by the Construction Department as the build progresses.

Regular monitoring of fire safety controls must be undertaken by Site and Contract Management – please see the inspection checklist references attached to this document

Table 1

| Property Type | Y/N | Separation Distance Note 1 | Timber Category Note 2 | Fire Extinguisher/Alarm Requirements Note 3 | Lighting Note 4 & 5 |
| --- | --- | --- | --- | --- | --- |
| 1 & 2 storey detached/semi-detached - Traditional build |  | N/A | N/A | Adequate numbers of suitable types of portable extinguishers must be made available throughout the site. As work progresses the adequacy of portable fire fighting equipment must be reviewed. Audible alarm must be in place. | N/A |
| 2.5 – 3 storey detached/semi-detached - Traditional build |  | N/A | N/A |  | N/A |
| 1 - 3 storey detached/semi-detached - Timber frame |  | Specific details to be identified on Site Layout Plan & Drawings |  |  | N/A |

Table 2

| Property Type | Y/N | Separation Distances & Sequence of Build Note 1 | Timber Frame Category Note 2 | Fire Extinguisher/ Alarm Requirement Note 3 | Lighting Note 4 & 5 | Fire brake/compartment requirements Note 6 & 7 |
| --- | --- | --- | --- | --- | --- | --- |
| Apartments/run of 3+ storey town houses – Traditional |  | N/A | N/A | Internal 1 per floor per core. Escape routes and exits to be marked and signed. Fire extinguishers sited on upper floors. Escape routes and exits to be marked and signed. Interlinked alarm system for Timber Frame build. | Adequate emergency lighting with back up to be installed (BS EN: 1838: 1999) | If travel distances are > than - Enclosed structure with 2 or more exits 45m - Enclosed structures with 1 exit (dead end) 18m - Haki type external stairs. |
| Apartments/runs of 3+ storey town houses –Timber Frame |  | Specific details to be identified on Site Layout Plan & Drawings |  |  |  | If travel distances are > than - Enclosed structure with 2 or more exits 25m - Enclosed structures with 1 exit (dead end) 12m - Haki type external stairs. |

Table Notes:

- Current Site Layout Drawings and Plans must be attached to this document. The details should also include the anticipated sequence of build.

- Timber frame category must be established using the Design Guide to Fire Safety in Construction produced by the Structural Timber Association (see flow chart below for quick reference).

- Adequate numbers of extinguishers must be provided. Contractors carrying out ‘Hot Works’ are to provide their own fire extinguisher which is appropriate to the work activity, being carried out. They must be correctly inspected and tested by the sub-contractor.

- Task/work lighting is to be provided by the sub-contractor, due to the hot nature Halogen lighting is not permitted on any Persimmon Group site.  All task/work lighting must be fluorescent 110v.  It must be correctly inspected and tested by the sub-contractor.

- If work carries on inside enclosed structures or at night, emergency escape lighting will be required to ensure that escape routes can be identified and used safely.

- Guidance on scaffold access and egress to be assessed and recorded on PHG/HS/038 (Specifications and safety requirements for working at heights).

- Maximum travel distances which are considered acceptable for a verity of situations.  The distances given are from the fire to an exit from the structure, typically a door leading to the outside at ground level, or to a stairway or compartment protected against fire.

- Consider internal temporary build electrics to be installed for use by contractors.

Construct Project as normal taking into consideration separation distances between timber frame build and occupied/ off-site neighbours.

Can this separation distance be achieved?

Construct project as normal.

Install fire engineered mitigation measures

or

build with a type B or C timber frame as defined in

STA Design Guide Part 3

or

Traditional Build construction

Yes

Yes

No

No

Refer to STA Design Guide Part 2 table 1 for separation distance required.

Note: As an alternative to this guidance, a more precise assessment by a competent fire engineer can be undertaken.

Is a single building over 600m2/6458ft2

total floor area?

PART A

Fire Safety Plan Details

Site Name:

Plan Prepared By (print name):

Signature:

Date:

| Site Based Responsible Persons |  |
| --- | --- |
| Appointment: | Name: |
| Site Fire Safety Coordinator |  |
| Site Assistant Fire co-ordinator |  |

| Alarms required |  |
| --- | --- |
| Type | Location |

| Compound Area |  |  |  |
| --- | --- | --- | --- |
| Fire Points Required |  | Extinguisher/Fire fighting Equipment Type | No |
| Offices | Provision of Welfare Facilities Procedure No: PHGPR:010 |  |  |
| Canteen |  |  |  |
| Flammable Store |  |  |  |
| Timber Store |  |  |  |
| Paint Store |  |  |  |
| LPG Store |  |  |  |
| Fuel Bund |  |  |  |
| Build Area |  |  |  |

| Site Temporary Build Electrical Supply – Compound Facilities Electrical installations, especially temporary ones, should be of sufficient capacity for the intended use and designed, installed, inspected and maintained by competent personnel. All site temporary electrical installations must be supported by a completion/test certificate. The installation must than be inspected every 3 months by a competent person (inspection certificate required). See construction manual for details |  |  |
| --- | --- | --- |
| External |  |  |
| Location | Instillation certificate Required | Dated provided |
| Site Office | YES |  |
| Site Welfare | YES |  |
| Silo | YES |  |
| Tower crane | YES |  |

| Inspections & Testing Recorded by the site management team on the site managers Weekly Check sheet PHG/HS/:014 |  |  |
| --- | --- | --- |
| Type | Frequency | Responsibility |
| Fire arrangements Inspected | Weekly |  |
| Fire Fighting Equipment | Weekly |  |
| Alarm Test (wired/wireless) | Weekly |  |
| Emergency Lighting | Weekly |  |

| Permits to Work ‘Permits to Work’ Procedure No: PHGPR:007 |  |
| --- | --- |
| Hot Work Activity Identified: | Yes/No |
| Plumbing (soldering) |  |
| Lead work |  |
| Hot roof work (if to be undertaken) |  |
| Welding and gas cutting |  |
| Abrasive wheels |  |

Contractors carrying out ‘Hot Works’ are to provide their own fire extinguisher which is appropriate to the work activity being carried out. They must be correctly inspected and tested by the sub-contractor.

SITE LAYOUT PLAN (IDENTIFY THE FOLLOWING) Assembly Point Location LPG Storage Area Location Flammable Material Storage Location Fuel Bund Re-fuelling area including any provision for re-fuelling of small plant

See also the traffic management plan

PART B - Apartment Block/Runs of 3+ storey town houses

Fire Safety Plan Details

Site Name:

Plan Prepared By (print name):

Signature:

Date:

| Provision of Site Fire Plan to Local Fire Services – if required under Paragraph 270 of HSG 168 |  |
| --- | --- |
| Contact Name: |  |
| Address |  |
| Tel: |  |
| Date issued: |  |

| Site Based Responsible Persons |  |
| --- | --- |
| Appointment: | Name: |
| Site Fire Safety Coordinator |  |
| Site Assistant Fire co-ordinator |  |

| Fire Brakes/Compartment Requirement Fire Brakes/Compartments will be required if the evacuation/travel distances in Table 2 above are exceeded, and cannot be complied with by the use of internal and/or external staircases. The Fire Brakes/Compartments should be constructed once the roof and internal staircases are in place and before internal fit out. |  |  |
| --- | --- | --- |
| Floor | Location | Requirement |
|  |  | Note: ceiling voids to be 30 min fire protected |

| Fire Point Requirement Fire points should be identifiable and located at each storey exit (i.e. a fire point per floor per stair core) |  |  |  |
| --- | --- | --- | --- |
| Floor | Location | Extinguisher/Fire fighting Equipment Type | Qty |

| Alarms required It is expected on apartment builds that an inter-connecting (could be protected wired-in or wireless) system of call-points and sounders will be required to provide an effective fire warning system. For example one that meets the requirements of BS 5839: 1:2002 + A2: 2008. See construction manual for details |  |
| --- | --- |
| Type | Location |

| Emergency Lighting Requirements Within buildings and enclosed structures, escape lighting (especially in escape routes) will generally be needed in the following circumstances. - Underground or windowless accommodation. - Stairs without natural, borrowed or spill lighting - Internal corridors without borrowed light, which is of sufficient length that the escape route would be unclear: and/or - Where work continues outside daylight hours. |  |  |
| --- | --- | --- |
| Total emergency lighting requirement for apartment block |  |  |

| Fire detection Required Detection may be needed in areas where a fire could start unnoticed. Consider commissioning the finished detection system as soon as practical. |  |
| --- | --- |
| Type | Location & Drawing reference |

| Site Temporary Build Electrical Supply – Apartment facilities Electrical installations, especially temporary ones, should be of sufficient capacity for the intended use and designed, installed, inspected and maintained by competent personnel. All site temporary electrical installations must be supported by a completion/test certificate. The installation must than be inspected every 3 months by a competent person (inspection certificate required). See construction manual for details. |  |  |
| --- | --- | --- |
| Internal (Apartment block) |  |  |
| Location | Instillation certificate Required | Dated provided |

| Signage Required: eg: No Smoking, No Naked Flames, Fire Exit, Fire Point, etc |  |  |  |
| --- | --- | --- | --- |
| Location | Type | Ser No | Qty |

Plans of apartment block to be attached showing locations of:

- Fire points

- Emergency lighting

- Temporary build electrics

- Emergency exits

- Emergency water access

Drawing details of fire brakes/compartments to be attached

FIRE PLAN LEGEND

| KEY |  |
| --- | --- |
| FIRE POINTS |  |
| EMERGENCY EXIT | + |
| FIRE EXIT SIGNS | + |
| EMERGENCY EXIT ROUTE |  |
| FIRE ALARM |  |
| TEMPORARY ELECTRICS | + |
| EMERGENCY LIGHTING |  |
| SCAFFOLD STAIRCASE |  |
| FIRE HYDRANT/EMERGENCY WATER SUPPLY |  |

Site Managers Timber Frame Safety Checklist

Is existing fire safety management plan current

Condition of existing fire breaks (particularly in the roof void)

Adequate storage of combustible materials to minimise fire loading

Adequate site hoarding /fencing to site

Establish assembly point

Control of hot work (permits)

Suitable fire detection and alarm system on each floor

Sufficient Emergency lighting

Sufficient number (and location) of fire extinguishers

Are all necessary directional signage within building

Are all laminated building fire layout plans posted at each fire point

Adequate number of escape routes (minimum of 2 required)

Adequate non combustible stairs as means of escape

Controls in place to prevent unauthorised entry after hours in areas

PLEASE INDICATE ANY ISSUES ON THE SITE MANAGERS WEEKLY CHECKLIST AND WHERE NECESSARY:

- CONTACT YOUR CONTRACT MANAGER

- LIAISE WITH GROUP HEALTH AND SAFETY ADVISOR

Contracts Managers Timber Frame Safety Checklist

Fire plan/log book - developed and maintained

All operatives, visitors, etc inducted in fire safety

Hot work permit system for residual hot works

Fire alarms and that fire stations are in place and operational

Suitable signage in place

Fire drills undertaken and results recorded

Safe storage of all flammable materials and LPG vessels

Housekeeping - All working areas are clean and tidy

PLEASE INDICATE ANY ISSUES ON THE CONTRACTS MANAGER MONTHLY CHECKLIST AND WHERE NECESSARY:

- LIAISE WITH GROUP HEALTH AND SAFETY ADVISOR

- LIAISE WITH LOCAL FIRE OFFICER AND EMERGENCY SERVICES IF NECESSARY